Lone Cabbage Oyster Reef Restoration Project:  Oyster Recruitment Monitoring April 2018-May 2019

Introduction:

Indices of recruitment are an important component of fish population assessments.  Applying stock-recruitment relationships to commercial shellfish species is confounded by the highly variable and prolific nature of spawning patterns.  Defining the stock-recruitment relationship in oysters is further complicated by the availability of hard settlement substrate; essential for oyster survival past the planktonic larval stage.  Here an oyster is considered recruited to the fishery post-settlement and post-cementation and these small oysters are referred to as spat.  This is when oysters are first able to be impacted by harvest activities.  Fishing mortality can occur to sub-market size oysters through field culling and sorting processes.

A minimal oyster recruitment monitoring program was established in order to provide basic information on oyster spawning patterns throughout the project area.  This program was coupled with a water quality monitoring program in order to increase efficiency of field operations, and also to enable association of continuous water quality data with observed recruitment patterns.  While developing a stock-recruitment relationship is currently beyond the scope of this monitoring program, it will provide a simple index of oyster recruitment.

Methods:

In order to monitor oyster recruitment, spat collectors were used.  Collector design and deployment time were initially unclear.  Shell spat collectors were deployed at each project water quality station in April-October 2018 (Figure 1).  Tile spat collectors have been deployed June 2018 to present.  The initial shell collector deployment time was two months.  Starting in June 2018, spat collectors were deployed for one month before being retrieved and replaced.  After retrieval, collectors are placed into zip-lock bags and frozen until processing.

Shell Spat Collectors:

Shell spat collectors consisted of ten clean market-size oyster shells enclosed in clam bag mesh (mesh size =).  Clean oyster shell was acquired from an oyster processing facility in Cedar Key, though it is not known how long harvested shell was dried prior to use.  After the collector was constructed, total volume (mL) was determined through displacement.  Total mass (g) was also collected.  Each shell spat collector was attached to water quality sensor housings, located immediately above the water bottom (Figure 2).

When processing, collectors were thawed and shell was removed from the mesh.  Length, width, height (mm) were recorded for each shell.  Total spat count for each side of shell (interior/exterior) was recorded, along with the height (mm) of the largest spat on each side.  Shell spat collectors were deployed April-October 2018, when it was decided only tile spat collectors would be used.

Tile Spat Collectors:

Tile spat collectors each consist of a 0.151 x 0.151 x  0.012 m (surface area = 0.058 m2).  A small hole was drilled in the center of each tile and secured to the water quality sensor anchor chain with a buoy so the tile was oriented horizontally 0.151m above the water bottom (Figure 2).

When processing, collectors were thawed and total spat count for each side of tile (top/bottom/side) was recorded.  The heights (mm) of the three largest spat per side were also recorded.  Tile spat collectors have been deployed June 2018 to present.

Supplemental data collected for both collectors included estimates of barnacle coverage and evidence of multiple spawning events (spat on spat) and predators (gastropod eggs).

Analysis:

Figures/Tables Checklist

X-Project Area Map

X-Collector deployment pic

X-Shell spat count total month station (convert to mass and volume)

X-Shell spat count mean side

X-Shell spat count mean side month

X-Shell spat count mean side month station

X-Shell spat size mean side

X-Shell spat size mean month

X-Shell spat size mean month station

X-Shell spat size vs shell dimensions

X-Barnacle coverage

X-Tile spat count total month station (convert to mass and volume)

X-Tile spat count mean side

X-Tile spat count mean side month

X-Tile spat count mean side month station

X-Tile spat size mean side

X-Tile spat size mean month

X-Tile spat size mean month station

X-Barnacle coverage

-Mean monthly salinity station

-Cumulative monthly salinity number readings >7ppt for each station

(Butler 1949, Loosanoff 1953…gametogenesis impaired below 5 or 6 ppt)

-Mean monthly temp

-Map of station distance from east pass

Normality Tests

ANOVA:  Spat Count differences by location month

GLMMs:  Environmental predictors of spat count

| Model | Fixed Effects | Random Effects |
| --- | --- | --- |
| 1 | -Mean monthly salinity -Mean monthly temperature -Distance from East Pass -Barnacle Coverage -Chl-a | Station Month Year |
| 2 | -Cumulative monthly salinity >X -Mean monthly temperature -Distance from East Pass -Barnacle Coverage -Chl-a | Station Month Year |
| 3 | -Better model from above plus side of reef |  |

Figure 1:  Location of the Lone Cabbage Reef elements and water quality stations.  Spat collectors were deployed on water quality sensor anchors.

Figure 2:  Shell bag (left) and tile (right) spat collectors deployed on water quality sensor anchor.

Results:

Shell Spat Collectors:

Figure:  Total spat counts observed on shell spat collectors by month and station from April 2018-October 2018.  Values have been standardized to unit shell mass (spat per 1 kg shell = total_kg) and unit shell volume (spat per 1 L shell = total_L).  Missing values indicate collectors were lost, or the station was removed during reef construction.

Figure:  Mean spat counts (+ standard error) observed on shell spat collectors by shell side from April 2018-October 2018.  Exterior side of the shell = ex_cnt.  Interior side of shell = in_cnt.

Figure:  Mean spat counts (+ standard error) observed on shell spat collectors by shell side and month from April 2018-October 2018.  Exterior side of the shell = ex_cnt.  Interior side of shell = in_cnt.

Figure:  Mean spat counts (+ standard error) observed on shell spat collectors by shell side, month, station from April 2018-October 2018.  Exterior side of the shell = ex_cnt.  Interior side of shell = in_cnt.  Missing values indicate collectors were lost, or the station was removed during reef construction.

Figure:  Mean maximum spat height (mm, + standard error) observed on shell spat collectors by shell side from April 2018-October 2018.  Exterior side of the shell = mx_ex_mm.  Interior side of shell = mx_in_mm.

Figure:  Mean maximum spat height (mm, + standard error) observed on shell spat collectors by shell side and month from April 2018-October 2018.

Figure:  Mean maximum spat height (mm, + standard error) observed on shell spat collectors by month and station from April 2018-October 2018.  Missing values indicate collectors were lost, the station was removed during reef construction, or no spat were observed.

Figure:  Relationship of total spat count to principal shell dimensions for shell spat collectors from April 2018-October 2018.  Shell height = sh_ht_mm.  Shell length = sh_ln_mm.  Shell width = sh_wd_mm.

Figure:  Barnacle coverage observed on shell spat collectors by tile side, month, station, and shell from April 2018-October 2018.  Missing values indicate collectors were lost, or the station was removed during reef construction.

Tile Spat Collectors:

Figure:  Total spat counts observed on tile spat collectors by month and station from June 2018-May 2019.  Missing values indicate tiles were lost, or the station was removed during reef construction.  Station WQ10 was established March 2019.

Figure:  Mean spat counts (+ standard error) observed on tile spat collectors by tile side from June 2018-May 2019.

Figure:  Mean spat counts (+ standard error) observed on tile spat collectors by tile side and month from June 2018-May 2019.

Figure:  Spat counts observed on tile spat collectors by tile side, month, and station from June 2018-May 2019.  Missing values indicate tiles were lost, or the station was removed during reef construction.  Station WQ10 was established March 2019.

Figure:  Mean maximum spat height (mm, + standard error) observed on tile spat collectors by tile side from June 2018-May 2019.

Figure:  Mean maximum spat height (mm, + standard error) observed on tile spat collectors by month from June 2018-May 2019.  Missing values indicate no spat were observed.

Figure:  Mean maximum spat height (mm, + standard error) observed on tile spat collectors by month and station from June 2018-May 2019.  Missing values indicate tiles were lost, the station was removed during reef construction, or no spat were observed.  Station WQ10 was established March 2019.

Figure:  Barnacle coverage observed on tile spat collectors by tile side, month, and station from June 2018-May 2019.  Missing values indicate tiles were lost, or the station was removed during reef construction.  Station WQ10 was established March 2019.

Spat Collector Type Comparison:

Figure:  Comparison of total spat counts on shell and tile spat collectors from April 2018-October 2018.  Shell totals have been standardized to unit shell mass (spat per 1 kg shell) and unit shell volume

(spat per 1 L shell).